NPK SENSOR

#include <SoftwareSerial.h>

#define RE 8

#define DE 7

const byte cmd[] = {0x01, 0x03, 0x00, 0x00, 0x00, 0x04, 0x44, 0x09};

byte response[13];

SoftwareSerial rs485(10, 11);

void setup() {

Serial.begin(4800);

rs485.begin(4800);

pinMode(RE, OUTPUT);

pinMode(DE, OUTPUT);

digitalWrite(RE, LOW);

digitalWrite(DE, LOW);

delay(3000);

Serial.println("🌱 ZTS-3002: Reading with noise tolerance");

}

void loop() {

// Flush buffer

while (rs485.available()) rs485.read();

// Transmit

digitalWrite(DE, HIGH);

digitalWrite(RE, HIGH);

delayMicroseconds(200);

rs485.write(cmd, sizeof(cmd));

rs485.flush();

digitalWrite(DE, LOW);

digitalWrite(RE, LOW);

// 🌟 Critical: Let RS485 bus settle before reading

delay(2); // Add 2ms settling time

delay(800); // Wait for response

// Read up to 20 bytes to capture possible noise

byte buffer[20];

int received = 0;

unsigned long start = millis();

while (received < 20 && (millis() - start) < 1000) {

if (rs485.available()) {

buffer[received++] = rs485.read();

}

}

// 🔍 Search for valid frame: look for 0x01 0x03 0x08

bool found = false;

int offset = -1;

for (int i = 0; i <= received - 13; i++) {

if (buffer[i] == 0x01 && buffer[i+1] == 0x03 && buffer[i+2] == 0x08) {

offset = i;

found = true;

break;

}

}

if (found) {

// Copy valid 13-byte frame

for (int i = 0; i < 13; i++) {

response[i] = buffer[offset + i];

}

uint16_t moisture = (response[3] << 8) | response[4];

int16_t temp      = (response[5] << 8) | response[6];

uint16_t ec       = (response[7] << 8) | response[8];

uint16_t ph       = (response[9] << 8) | response[10];

Serial.println("🌱 Soil Readings:");

Serial.print("Moisture: "); Serial.print(moisture / 10.0, 1); Serial.println(" %");

Serial.print("Temp:     "); Serial.print(temp / 10.0, 1); Serial.println(" °C");

Serial.print("EC:       "); Serial.print(ec); Serial.println(" µS/cm");

Serial.print("pH:       "); Serial.print(ph / 10.0, 1); Serial.println(" ");

} else {

Serial.println("🌱 Soil Readings:");

Serial.println("Moisture: ❌");

Serial.println("Temp:     ❌");

Serial.println("EC:       ❌");

Serial.println("pH:       ❌");

Serial.print("Raw: ");

for (int i = 0; i < received; i++) {

if (buffer[i] < 0x10) Serial.print("0");

Serial.print(buffer[i], HEX); Serial.print(" ");

}

Serial.println();

}

Serial.println("---");

delay(3000);

}